# NaoDec Media Playback Controller Build and Max Setup

# NaoDec Media Playback Controller Build and Max Setup

Revision: 2.2
Date: 2026-07-02
Controller: ESP32-S3 Wi-Fi OSC controller for Max/MSP media playback
Schematic: NaoDec_Media_Playback_Controller_Schematic_Rev2.0.html

Rev 2.2 — Added an “R1/R2/C1/C2 Fallback (Troubleshooting Reference)” note under the encoder section: build with those four parts unpopulated, and only fit them (10 kOhm / 100 nF, matching the button RC values) if the KY-040 shows encoder misreads that firmware filtering alone doesn’t resolve.

Rev 2.1 — Added an explicit placement requirement: U1 and its support PCB must be mounted inside the operator panel enclosure, not staged near the router or any other remote equipment. See Section 1.

Rev 2.0 — Design hardening: LED1 Wi-Fi status indicator (GPIO12 via R11 330 Ω), R6–R10 100 Ω series protection on every off-board GPIO line, H1 keyed JST-XH 5-pin encoder connector, encoder shaft-to-GND continuity check, USB-C cable spec ≤ 1 m. Also folds in the KY-040 optional encoder-replacement documentation. Rev 1.0 schematic remains in the repo for reference.

Rev 1.1 — Sections 6 and 8 aligned with the real ASUS RT-AX1800HP LAN: Max host 192.168.50.2, controller reserved at 192.168.50.114 per the .11x convention.

## 1. Purpose

This controller is a small operator panel for a Max/MSP media player. It does not carry audio, video, LED data, or LED power. It only sends control messages over Wi-Fi using OSC over UDP.

Functions:

| Control | Action in Max |
| --- | --- |
| Volume encoder | Sends /volume <0.0...1.0> |
| Play button | Sends /transport/play 1 |
| Pause button | Sends /transport/pause 1 |
| Stop button | Sends /transport/stop 1 |

The Max computer should be connected to the ASUS router by wired Ethernet. The ESP32-S3 connects to the same LAN by 2.4 GHz Wi-Fi.

### Placement Requirement

U1 (the ESP32-S3) and its support PCB must be mounted inside the operator panel enclosure, next to the encoder and buttons it drives. Do not place U1 near the router, the Mac Mini, or any other equipment and run long wires out to a separate panel.

The wire schedule in the schematic specifies a maximum run of about 0.5 m for every panel signal (encoder A/B, Play/Pause/Stop, status LED). These are unbuffered 3.3 V logic lines: a switch-to-GND button line or a quadrature encoder line stretched to several meters acts as a noise antenna, and R6-R10 (100 Ohm) only provide ESD/ringing protection for short runs, not EMI immunity over distance. Extending these runs to meters of cable, especially near a Wi-Fi router or switching power supplies, risks phantom button presses, missed presses, and misread encoder detents.

There is no benefit to placing U1 near the router: the whole point of the Wi-Fi OSC design is that the long leg of the signal path (to the Max computer) does not need wires. 2.4 GHz Wi-Fi easily covers the distance between the operator panel and the router. Keep U1 at the panel and let Wi-Fi carry the distance; run only a USB-C power cable to the panel location.

If a project constraint truly requires the microcontroller to sit apart from the panel by more than about 0.5 m, that is a different design (a remote I/O interconnect over UART/RS-485 or a buffered I2C expander) and is out of scope for this Rev 2.0 hardware.

## 2. Required Items

| Ref | Qty | Item | Description |
| --- | --- | --- | --- |
| U1 | 1 | ESP32-S3-DevKitC-1 | Main controller board, USB-C, N8R8 or N16R8 preferred |
| ENC1 | 1 | NEBDS-01 / EC11 encoder module | Preferred rotary encoder module with Schmitt trigger / anti-bounce, pins A, B, SW, VCC, GND; KY-040 may be used as an optional replacement |
| SW1-SW3 | 3 | Normally-open momentary pushbutton | Play, Pause, Stop panel buttons |
| R3-R5 | 3 | 10 kOhm resistor | Pull-ups for Play/Pause/Stop buttons |
| R6-R10 | 5 | 100 Ohm resistor | Series protection between each GPIO and its panel wire (ESD / short protection, edge damping) |
| R11 | 1 | 330 Ohm resistor | Series resistor for LED1 status LED |
| LED1 | 1 | Panel-mount LED, 3-5 mm | Wi-Fi status indicator on the operator panel |
| C3-C5 | 3 | 100 nF capacitor | Button debounce capacitors, X7R ceramic, 25 V or higher |
| J1 | 1 | USB-C cable | Power cable for ESP32-S3 board, quality cable, 1 m or shorter |
| PSU1 | 1 | USB power adapter | Regulated 5 V, 2 A wall adapter |
| PCB1 | 1 | Prototype PCB | 2.54 mm prototype PCB or custom PCB, approx. 70 x 50 mm |
| ENCLOSURE | 1 | Nonconductive enclosure | Plastic controller box with strain relief and ventilation |
| H1 | 1 | JST-XH 5-pin connector set | Keyed connector for the encoder harness (mis-plug-proof) |
| H2-H5 | 4 | 2-pin locking connectors | Front-panel connectors for buttons and LED1 |
| W1 | 1 lot | Hookup wire | 24 AWG stranded wire, 7 colors, 300 V insulation or better |
| HW1 | 1 lot | Assembly hardware | Standoffs, M3 screws, encoder knob, labels, heat-shrink tubing |

Recommended spares:

| Item | Qty |
| --- | --- |
| NEBDS-01 / EC11 encoder module, or KY-040 module for bench/prototype replacement | 1 |
| Momentary pushbutton | 1 |
| 10 kOhm resistor | 2 |
| 100 Ohm resistor | 2 |
| 100 nF capacitor | 2 |
| Panel-mount LED | 1 |

## 3. Pin Connections

### ESP32-S3 Pin Map

| Function | ESP32-S3 Pin | Wire Color | Notes |
| --- | --- | --- | --- |
| Encoder A | GPIO7 | Green | To ENC1 A (or CLK on KY-040) through R6 (100 Ohm) |
| Encoder B | GPIO8 | Blue | To ENC1 B (or DT on KY-040) through R7 (100 Ohm) |
| Play | GPIO9 | White | Button input, active low, through R8 (100 Ohm) |
| Pause | GPIO10 | Yellow | Button input, active low, through R9 (100 Ohm) |
| Stop | GPIO11 | Orange | Button input, active low, through R10 (100 Ohm) |
| Status LED | GPIO12 | Grey | To LED1 anode through R11 (330 Ohm), active high |
| Encoder/module power | 3V3 | Red | Connect to ENC1 VCC on NEBDS-01, or + on KY-040 |
| Ground | GND | Black | Common ground for encoder, buttons, and LED1 |
| Power input | USB-C | USB cable | Use USB-C input path, not GPIO or 3V3 pin |

Every signal that leaves the PCB (GPIO7-GPIO11) passes through a 100 Ohm series resistor (R6-R10) mounted on the PCB, close to the ESP32. These protect the GPIOs against ESD and wiring shorts and damp reflections on the panel runs, regardless of which encoder module is fitted.

### Encoder Module: NEBDS-01 Preferred, KY-040 Optional

| Function | NEBDS-01 / EC11 Schmitt Pin | KY-040 Pin | ESP32-S3 Connection |
| --- | --- | --- | --- |
| Encoder channel A | A | CLK | GPIO7 via R6 (100 Ohm) |
| Encoder channel B | B | DT | GPIO8 via R7 (100 Ohm) |
| Encoder switch | SW | SW | Not connected |
| Module power | VCC | + | ESP32 3V3 |
| Ground | GND | GND | ESP32 GND |

The encoder harness terminates in H1, a keyed JST-XH 5-pin connector (A, B, VCC, GND, plus one spare position for SW). A single keyed connector makes it impossible to swap power against signal pins at reassembly.

Do not add external 100 nF capacitors to encoder A or B when using this Schmitt trigger module. The original bare-encoder RC parts R1, R2, C1, and C2 are not populated for this build.

KY-040 optional replacement notes:

| Topic | NEBDS-01 / EC11 Schmitt Module | KY-040 Optional Replacement |
| --- | --- | --- |
| Signal conditioning | Schmitt trigger / anti-bounce output | Usually raw or lightly pulled-up mechanical encoder output |
| Wiring | A -> R6 -> GPIO7, B -> R7 -> GPIO8, VCC -> 3V3, GND -> GND, SW -> NC | CLK -> R6 -> GPIO7, DT -> R7 -> GPIO8, + -> 3V3, GND -> GND, SW -> NC |
| External A/B RC parts | Do not populate R1, R2, C1, or C2 | Leave R1, R2, C1, and C2 unpopulated first; use firmware filtering and test rotation behavior |
| Firmware requirement | Normal quadrature validation | Stronger transition validation / debounce is recommended |
| Best use | Final installed controller | Bench test or replacement when NEBDS-01 is unavailable |

Power any module-style encoder from ESP32 3V3, not 5 V. If the KY-040 is powered from 5 V, its CLK, DT, or SW outputs may also pull up to 5 V, which is not safe for ESP32-S3 GPIO. R6/R7 (100 Ohm) apply to either module and do not substitute for using the correct 3.3 V supply.

The NEBDS-01 / EC11’s metal frame and shaft normally ground through the module mounting pins. After mounting, verify continuity from the encoder shaft to GND — the knob is the most-touched, most ESD-exposed part of a plastic enclosure. Most KY-040 breakout boards use a plastic shaft with no frame ground path; this check applies only if the fitted module has a metal shaft or frame.

### R1 / R2 / C1 / C2 Fallback (Troubleshooting Reference)

Build the controller with R1, R2, C1, and C2 unpopulated. Do not fit them by default, with either encoder module.

Only revisit this if you observe encoder misreads in testing: the volume jumps by more than one step per detent, skips steps, or reads the wrong direction intermittently. This is a symptom of unfiltered mechanical contact bounce that firmware filtering alone did not fully clean up, and it is only expected with the KY-040 (bare mechanical contacts). It should not happen with the NEBDS-01 (Schmitt trigger conditioning is already onboard).

| Module | Default | If encoder misreads occur |
| --- | --- | --- |
| NEBDS-01 / EC11 | R1/R2/C1/C2 unpopulated | Do not populate — the module’s onboard Schmitt trigger already conditions the signal. Look for a wiring or grounding fault instead. |
| KY-040 | R1/R2/C1/C2 unpopulated; rely on firmware filtering first | Populate R1 = R2 = 10 kOhm (to 3V3) and C1 = C2 = 100 nF (to GND) — the same values as R3-R5/C3-C5 on the buttons, giving the same ~1 ms RC debounce constant |

### Play / Pause / Stop Buttons

Each button is a normally-open switch from its GPIO input to ground.

| Button | GPIO | Pull-Up | Debounce Capacitor | Series Resistor | Switch Wiring |
| --- | --- | --- | --- | --- | --- |
| Play | GPIO9 | R3, 10 kOhm to 3V3 | C3, 100 nF to GND | R8, 100 Ohm | Button closes the line to GND |
| Pause | GPIO10 | R4, 10 kOhm to 3V3 | C4, 100 nF to GND | R9, 100 Ohm | Button closes the line to GND |
| Stop | GPIO11 | R5, 10 kOhm to 3V3 | C5, 100 nF to GND | R10, 100 Ohm | Button closes the line to GND |

The pull-up and debounce capacitor sit at the GPIO node on the PCB; the 100 Ohm series resistor sits between that node and the panel wire to the button.

### Status LED (LED1)

| Item | Connection |
| --- | --- |
| LED1 anode | GPIO12 through R11 (330 Ohm), active high |
| LED1 cathode | GND (operator-panel ground return) |
| Drive current | ~4 mA from the 3.3 V GPIO — safe for any GPIO, bright enough indoors |

| LED state | Meaning |
| --- | --- |
| Solid on | Wi-Fi connected, controller operational |
| Blinking | Wi-Fi disconnected, reconnect in progress |
| Off | No power (or firmware fault) |

## 4. Build Steps

- Drill the enclosure for the encoder, three buttons, the status LED, USB cable, and any service access.

- Arrange controls left to right as VOLUME, PLAY, PAUSE, STOP, with LED1 visible from the operator position.

- Mount the ESP32-S3 board inside this same enclosure so the antenna end faces plastic, not metal. Do not mount U1 in a separate location and run extension wires to the panel (see Section 1, Placement Requirement).

- Keep at least 10 mm clearance around the ESP32 antenna area.

- Mount the NEBDS-01 / EC11 encoder module and fit the knob. If using the optional KY-040 module, mount it in the same volume encoder position.

- Mount the three normally-open pushbuttons and LED1.

- Build the 3.3 V and GND buses on the prototype PCB.

- Install R3-R5 from 3.3 V to GPIO9/GPIO10/GPIO11.

- Install C3-C5 from GPIO9/GPIO10/GPIO11 to GND.

- Install the series resistors on the PCB, close to the ESP32: R6-R10 (100 Ohm) in line with GPIO7-GPIO11, and R11 (330 Ohm) in line with GPIO12.

- Wire encoder A, B, VCC, and GND to the ESP32 through the keyed H1 JST-XH connector.

- Leave encoder SW unconnected and insulated (route it to the spare H1 position if you want it available later).

- Wire each pushbutton between its series resistor output and GND.

- Wire LED1 anode to the R11 output and LED1 cathode to GND.

- Verify continuity from the encoder shaft/frame to GND after mounting.

- Add strain relief for the USB-C cable. Use a quality cable no longer than 1 m.

- Label both ends of every wire.

- Inspect for solder bridges, loose strands, reversed connectors, and accidental contact near the antenna.

## 5. Electrical Checks Before Firmware

Perform these checks before plugging the controller into a computer or router setup.

| Check | Method | Pass Condition |
| --- | --- | --- |
| Visual inspection | Inspect PCB and connectors | No shorts, loose strands, or unlabeled harnesses |
| Unpowered resistance | Measure 5 V-to-GND and 3.3 V-to-GND | No near-short |
| Series resistors | Measure from each GPIO pad to its panel-side point, unpowered | ~100 Ohm (GPIO7-11), ~330 Ohm (GPIO12) — not 0 Ohm |
| Encoder shaft ground | Measure encoder shaft/frame to GND | Continuity (near 0 Ohm) |
| 3.3 V rail | Power by USB-C and measure 3V3 to GND | 3.20 V to 3.40 V |
| Encoder module VCC | Measure ENC1 VCC or KY-040 + to GND | 3.20 V to 3.40 V |
| Button idle voltage | Release Play/Pause/Stop and measure GPIO9-11 | Above 3.0 V |
| Button active voltage | Press each button and measure its GPIO | Below 0.3 V |
| Encoder A/B level | Rotate encoder and measure A/B, or KY-040 CLK/DT | Logic should stay within 0 V to 3.3 V |
| Status LED | Drive GPIO12 high (or run test firmware) | LED1 lights; current ~4 mA |

Do not apply 5 V to GPIO7, GPIO8, GPIO9, GPIO10, GPIO11, GPIO12, or the encoder module signal pins.

## 6. Firmware Configuration

Store Wi-Fi and Max destination settings in a local config file that is excluded from Git.

Project network assignment:

| Device / Setting | Value |
| --- | --- |
| Media playback controller reserved IP | 192.168.50.114 |
| Max player computer IP | Confirm from the ASUS router setup |
| OSC UDP port | 9000 |

The controller IP 192.168.50.114 should normally be assigned by ASUS DHCP reservation using the ESP32 MAC address. MAX_HOST is different: it must be the IP address of the Max player computer that receives OSC.

WIFI_SSID=ASUS_NAODEC
WIFI_PASSWORD=<see router credentials reference — not stored in this file>
MAX_HOST=192.168.50.2
MAX_OSC_PORT=9000

Firmware behavior required for Rev 2.0:

| Area | Requirement |
| --- | --- |
| Wi-Fi | Connect by DHCP and reconnect automatically after loss |
| Logging | Report assigned IP, Max host, and events over USB serial |
| Credentials | Never print the Wi-Fi password |
| Buttons | Send one OSC command on the debounced press edge only |
| Encoder | Decode valid quadrature transitions from GPIO7/GPIO8; apply stronger filtering if KY-040 is used |
| Volume | Maintain integer volume 0...100 |
| Volume step | Change volume by 2 percentage points per detent |
| OSC volume | Send normalized float 0.0...1.0 |
| Limits | Clamp volume between 0.0 and 1.0 |
| Memory | Save last volume after about 2 seconds of inactivity |
| Boot | Do not send Play, Pause, or Stop on boot |
| Status LED | GPIO12 solid high when Wi-Fi is connected; blink at ~2 Hz while disconnected or reconnecting |

## 7. OSC Message Contract

These OSC messages are the Rev 2.0 proposed contract (unchanged from Rev 1.0). Confirm that the final Max patch uses the same addresses and UDP port before acceptance testing.

| Function | OSC Address | Argument | Example |
| --- | --- | --- | --- |
| Play | /transport/play | integer 1 | /transport/play 1 |
| Pause | /transport/pause | integer 1 | /transport/pause 1 |
| Stop | /transport/stop | integer 1 | /transport/stop 1 |
| Volume | /volume | float 0.0...1.0 | /volume 0.65 |

Default UDP destination port: 9000.

## 8. ASUS Router Setup

This project’s ASUS RT-AX1800HP is already configured per the NaoDec network setup guide. Reuse that configuration rather than creating a new one.

Router proximity is a network concern only. It has no bearing on where U1 is physically mounted — U1 belongs inside the operator panel enclosure regardless of how far that panel sits from the router (see Section 1, Placement Requirement). 2.4 GHz Wi-Fi range easily covers typical operator-panel-to-router distances.

- 2.4 GHz SSID ASUS_NAODEC is enabled (see the router credentials reference for the password; do not commit it to this repo).

- The Max computer (Mac Mini) connects to an ASUS LAN port by Ethernet, static IP 192.168.50.2, with the Router field left blank so the Mac keeps using its Wi-Fi interface for internet (see the network setup guide, section 3).

- The ESP32 Wi-Fi network and the wired Max computer share the same LAN, gateway 192.168.50.1.

- Wireless client isolation / AP isolation must remain disabled for this SSID.

- DHCP pool is 192.168.50.100-192.168.50.199; the Max computer’s 192.168.50.2 is a static address outside the pool, not a reservation.

- Reserve a DHCP address for the media playback controller by MAC address under LAN -> DHCP Server -> Manually Assigned IP, at 192.168.50.114.

- Record the controller’s MAC address and confirm the reservation with ping 192.168.50.114.

Addressing for this LAN:

| Device | IP |
| --- | --- |
| ASUS RT-AX1800HP (gateway) | 192.168.50.1 |
| Max computer (Mac Mini, Ethernet, static) | 192.168.50.2 |
| Media playback controller (this build, DHCP reservation) | 192.168.50.114 |

192.168.50.114 follows the existing NaoDec convention of assigning each new ESP32 the next .11x address (.111, .112, .113 are already in use by other NaoDec ESP32 modules).

Do not set MAX_HOST to 192.168.50.114; that address belongs to the ESP32 controller. Set MAX_HOST to the Max player computer’s IP (192.168.50.2).

## 9. Connect to Max/MSP

The Max computer receives OSC over UDP from the ESP32 controller. Use a receive chain like this:

[udpreceive 9000]
       |
   [oscparse]
       |
[route transport volume]

Recommended routing:

[route transport volume]
      |          |
      |          +--> volume float 0.0...1.0 -> clamp -> master gain
      |
      +--> [route play pause stop]
              |     |      |
              |     |      +--> stop and reset player
              |     +--------> pause player
              +--------------> start player

Map messages this way:

| Incoming Message | Max Action |
| --- | --- |
| /transport/play 1 | Start or resume media playback |
| /transport/pause 1 | Pause playback |
| /transport/stop 1 | Stop playback and reset according to patch behavior |
| /volume 0.0...1.0 | Clamp, then drive media-player master gain |

Keep media-player volume separate from NeoDDP global brightness. They may both use normalized values, but they control different things.

## 10. Max Test Patch Checklist

Before connecting to the real media player actions, test with prints first.

- Add [udpreceive 9000].

- Connect it to [oscparse].

- Connect that to [print OSC_IN].

- Power the ESP32 controller.

- Press Play once and confirm one /transport/play 1 event.

- Press Pause once and confirm one /transport/pause 1 event.

- Press Stop once and confirm one /transport/stop 1 event.

- Rotate the encoder clockwise and confirm volume increases.

- Rotate the encoder counterclockwise and confirm volume decreases.

- Only after the printed messages are correct, connect them to the player controls.

If Max receives nothing:

| Symptom | Check |
| --- | --- |
| No UDP messages | Confirm ESP32 MAX_HOST is the Max computer IP, not 192.168.50.114 |
| Wrong port | Confirm both firmware and Max use UDP 9000 |
| ESP32 not reachable | Confirm same LAN/VLAN and AP isolation disabled |
| Duplicate button messages | Increase firmware debounce or check switch wiring |
| Encoder direction reversed | Swap A/B wiring or reverse direction in firmware |
| Volume affects lights | The Max patch is mapped to NeoDDP brightness instead of media gain |

## 11. Acceptance Tests

Run these before calling the controller ready.

| ID | Test | Pass Condition |
| --- | --- | --- |
| ELEC-01 | Visual inspection | No shorts, loose wires, or exposed conductors |
| ELEC-02 | 3.3 V rail | 3.20 V to 3.40 V |
| ELEC-03 | Encoder shaft ground | Continuity from shaft/frame to GND |
| NET-01 | Wi-Fi join | Controller gets DHCP address 192.168.50.114 within 15 seconds |
| NET-02 | Max reachability | Max computer and ESP32 are on same LAN/VLAN |
| NET-03 | UDP receive | Max receives only on the configured IP and port |
| NET-04 | Reconnect | Controller reconnects after Wi-Fi/router interruption |
| FUNC-01 | Play | One press sends one play command |
| FUNC-02 | Pause | One press sends one pause command |
| FUNC-03 | Stop | One press sends one stop command |
| FUNC-04 | Held button | Holding button for 3 seconds sends no repeats |
| FUNC-05 | Volume clockwise | Ten detents increase by about 20 percentage points |
| FUNC-06 | Volume counterclockwise | Ten detents decrease by about 20 percentage points |
| FUNC-07 | Volume limits | Output remains clamped from 0.0 to 1.0 |
| FUNC-08 | Boot behavior | No Play, Pause, or Stop command on power-up |
| FUNC-09 | Status LED | LED1 solid within 15 s of power-up; blinks when the router 2.4 GHz radio is disabled; returns to solid after re-enable (ties into NET-04) |
| SYS-01 | Site range | Reliable control at final operator location |
| SYS-02 | Soak | Stable operation for 8 hours with no unintended command |

## 12. Commissioning Record

Fill this out for each built controller.

| Field | Value |
| --- | --- |
| Controller serial / asset ID |  |
| ESP32-S3 board variant |  |
| Firmware version / commit |  |
| ASUS SSID |  |
| Controller MAC address |  |
| Controller reserved IP | 192.168.50.114 |
| Max computer IP |  |
| OSC UDP port |  |
| Max patch filename / revision |  |
| PSU make / model |  |
| Test date |  |
| Tested by |  |
| Result / open issues |  |

## 13. Release Gate

The controller is ready for installation only when:

| Gate | Required Result |
| --- | --- |
| Electrical tests | Pass |
| Network tests | Pass |
| Max OSC contract | Confirmed against the actual Max patch |
| Functional tests | Pass |
| Site range test | Pass |
| Enclosure | Closed, labeled, strain-relieved, and free of exposed conductors |
| Documentation | Commissioning record complete |